|  | - COOMET Recommendation | CООМЕТ R/IT/XX:202Y |
| --- | --- | --- |
|  | - Requirements for the Information and Communication Infrastructure to Ensure the Information Security of the COOMET Digital Platform |  |
| - Approved at ___ COOMET Committee meeting (_________________) |  |  |

1. SCOPE OF APPLICATION

The present Recommendation is developed in development of the COOMET Concept on Digital Transformation in of metrology (Program COOMET R8/2023) and defines the requirements to the information and communication infrastructure to ensure information security of the COOMET digital platform, which should be taken into account during its development.

This Recommendation applies to the modules of information security tools (IST), applied in the information and communication infrastructure of COOMET digital platform.

These modules are software means of protection against unauthorized access, designed to protect information that does not contain information constituting a state secret, as well as to ensure the security of personal data in information systems.

2. REFERENCES

The following publications are referenced in this Recommendation:

COOMET R8/2023 COOMET Concept on Digital Transformation in Metrology.

3. TERMS AND ABBREVIATIONS

3.1 The following terms with corresponding definitions are used in this recommendation:

Information and Communication Infrastructure - a set of technical and software tools, technologies, standards and protocols ensuring the creation, transfer, processing, use, storage, protection and destruction of information.

COOMET Digital Platform (Platform) - COOMET information resource.

3.2 The following abbreviations are applied in this Recommendation:

IT - Information Technologies

IST -  Information security tools

4. GENERAL PROVISIONS

4.1 Purpose of the information protection systems of the COOMET Digital Platform

The main purpose of the IST Platform applied in the information and communication infrastructure of the Platform is:

- Preventing unauthorized access, copying, provision or distribution of restricted information (personal data, confidential information of a commercial or official secret);

- exclusion unauthorized destruction or modification of information (ensuring the integrity of information);

- exclusion of unlawful blocking (actions resulting in artificial obstruction of access to information) of restricted information (ensuring accessibility of information);

- ensuring compliance with the requirements for the protection of information and personal data in information systems, established by the legislation in the place of registration of the operator of the COOMET digital platform.

4.2 Requirements to the structure and functions of the IST

As part of the IST, the following shall be implemented:

- a subsystem for managing access and user credentials;

- a subsystem for registering security events;

- a subsystem for ensuring integrity;

- an antivirus protection subsystem (by means of infrastructure).

The platform shall perform the following functions:

- functions of identification and authentication of access subjects of the Platform;

- functions of access control to the Platform objects;

- functions of security event registration.

A role-based access control method shall be implemented, providing for role-based access control to objects. Access subjects are understood as Platform users.

The list of roles of access subjects of the Platform includes:

- internal user roles - roles used to control access of internal users of the Platform;

- anonymous role ("Guest") - a role used to manage user access to the Platform resources that do not require authentication.

The lifecycle of internal and external user accounts is managed through the mechanisms of the IST Platform.

An access type is defined for each role.  An access type is a permissible subset of the types of operations (actions), allowed to be performed by an access subject (in accordance with the role) when accessing access objects.

The Platform shall define set of operation types that can be performed by access subjects when accessing access objects:

- reading data;

- adding data

- modification (change) of data;

- deletion of data.

Access objects are predefined sets of static and dynamic web pages, as well as individual web pages that contain protected information (including personal data, data on the settings and configuration of the IST mechanisms, etc.) intended for display to the user and subsequent processing by the user of the Platform.

4.3 Requirements for access control facilities

4.3.1 Identification and authentication of internal users

IPS shall identify and authenticate system users who are internal users of the Platform (internal users) and processes run on behalf of these users. Users must be uniquely identified and authenticated for all types of access, except for access to Platform resources that do not require identification and authentication (e.g., read access to the public section of the Platform's Internet portal).

User authentication shall be performed using passwords.

IST Platforms shall be able to unambiguously map a user ID to processes running on the user's behalf.

4.3.2 Identifier management

IST Platforms should perform the following user ID management functions:

1) ensuring the uniqueness of the user identifier;

2) assigning an identifier to a user;

3) blocking the user identifier after a set period of non-use.

4.3.3 Management of authentication means

IST Platforms shall perform the following functions of managing the means of authentication (authentication information) of users:

1) generation and issuance of initial authentication information (user password);

2) setting password characteristics:

a) setting the minimum password complexity with defined requirements for case, number of characters, combination of upper and lower case letters, digits and special characters;

b) setting the password validity time;

c) prohibiting users from using the number of last used passwords defined by the operator when creating new passwords;

Since the IST provides for the use of a password-based authentication mechanism, its recommended characteristics are given below:

- password length not less than six characters;

- password alphabet of at least 60 characters;

- maximum number of unsuccessful authentication attempts (entering an incorrect password) before blocking from 3 to 10 attempts;

- automatic blocking of the user account if the set maximum number of unsuccessful authentication attempts is reached from 5 to 30 minutes.

3) blocking (termination) and replacement of lost or compromised or damaged authentication means;

4) setting the maximum number of unsuccessful authentication attempts (entering an incorrect password) before user account lockout;

5) automatic blocking of the user account in case of reaching the set maximum number of unsuccessful authentication attempts;

6) protection of authentication information from unauthorized access to it and modification.

4.4 Requirements to the security event registration subsystem

The Platform security event registration subsystem can be implemented by both technical and organizational measures and shall provide:

- registration of user entry (exit) to (from) the system or registration of loading and initialization of the operating system and its software stop;

- recording of the following parameters in the logs

- the date and time of the user logging in (logging out) or loading (stopping) the system, the result of the login attempt (successful or unsuccessful);

- the user identifier presented during the access attempt;

- accounting of protected data carriers;

- protection of information about security events.

4.5 Requirements for the integrity assurance subsystem

The integrity assurance subsystem of the Platform shall provide integrity control of software, including software of information protection means.

The following organizational measures shall be implemented in the integrity assurance subsystem:

- determination of the circle of persons who are allowed to make changes to the configuration of information systems and information protection systems;

- documentation of information (data) about changes in the configuration of information systems and information protection systems;

- analysis of the potential impact of planned changes to the configuration of information systems and information protection systems on information security;

management of configuration changes of information systems and information security systems.

4.6 Requirements for the user access and credentials management subsystem

In terms of user account management, the subsystem of access management and user credentials of the Platform shall provide:

- possibility of forming a single catalog of user credentials;

- administration of account life cycle: automatic creation, updating, locking, unlocking;

- management of account locking and unlocking.

In terms of user access rights management, the subsystem shall provide:

- the ability to automate procedures for requesting and coordinating access rights to information resources;

- automatic assignment of authorizations to users in the protected information systems in accordance with the approved requests for access rights.

4.7 Requirements for operation modes

The operating modes of the Platform shall:

- Ensure the possibility of round-the-clock operation of the IST Platform;

- ensure independence of the IST from changes in the organizational structure of the Platform while preserving the composition and content of the functions performed;

- allow setting and changing the configuration of information protection means without reprogramming;

- provide the ability to change the settings, policies and rules of user access to the IST in case of changes in the IT infrastructure, changes in information security regulations;

- provide the ability to identify the causes of malfunction using the journaling functions.

4.8 Requirements for destination indicators

The architecture of the IST Platform shall not impose any restrictions on the information technologies used in the Platform.

The architecture of the IST Platform shall ensure the realization of information protection functions at all technological stages of the IST, including during maintenance and repair.

4.9 Reliability requirements

The hardware and software infrastructure to ensure information security of the Platform shall be designed to operate in a round-the-clock mode and allow for the implementation of redundancy procedures and system recovery after failures.

4.10 Security requirements

Placement and installation of the hardware and software used to ensure information security of the Platform shall be performed in strict compliance with the requirements established by the manufacturers of these technical means.

Placement of the equipment at standard locations shall ensure its safe maintenance and operation.

Under normal conditions, technical means and individual units shall not lose physical stability to such an extent as to endanger users and service personnel.

The design of the technical means shall limit the risk of fire or electric shock due to electrical or mechanical overloads, failures, abnormal operation or operational errors.

Technical means shall provide protection of users and service personnel from electric shock.

The values of harmful factors of the production environment and labor process when working with technical means (levels of electromagnetic fields, acoustic noise, concentration of harmful substances in the air, soft X-ray radiation), which have an adverse effect on human health, shall not exceed the current norms.

4.11 Requirements for patent purity

When upgrading hardware and software to ensure information security of the Platform, copyrights must be observed.

When supplying the software, the requirements regarding the transfer of a non-exclusive license must be met.

Fulfillment of the requirements for software license purity is ensured by the developer of the COOMET Digital Platform.

Software purchased from third-party organizations must be accompanied by documentation confirming the competence of these organizations to supply this product and accompanied by a license agreement.

4.12 Requirements for standardization and unification

Solutions for the use of hardware and software to ensure information security of the Platform shall, preferably, consist of identical components in order to reduce maintenance and repair costs, interchangeability of used components, and ease of operation.

4.13 Requirements for operation

The conditions, operating mode, types and frequency of maintenance of technical means are determined by the requirements of suppliers of technical means and software.

INFORMATION DATA

COOMET Recommendation R/IT/XX:202Y

1. Development coordinator: COOMET Task Group on Digital Transformation in Metrology (TG-DigTr)

2. COOMET Project: 889/RU/23 (coordinator - Andrey Pankov, VNIIMS, Russia, Chairman of TG-DigTr)

3. The recommendation was approved at ___ COOMET Committee meeting ( )

Since this COOMET Recommendation has organizational and methodological content and reflects the issues of COOMET information resources functioning, this Recommendation is applied by all participants of cooperation within COOMET.